B	Mathematics of the model

A complete mathematical specification of the urban water model is presented in this appendix.

All variables in the model are positive (i.e. greater than or equal to zero). The variables vRestr and vRestr0 are binary: taking only a value of 0 or 1. Investment variables may either be binary or continuous, depending on the nature of the supply source.

Equations for the basecase, market model, representing flexible prices are presented first (B.1). Policy interventions are modelled by constraining the basecase model according to the equations listed in section B.2. All sets, parameters and variables in the model are detailed in section B.3.

## B.1	Core market model

### Objective function

(B.1)

Objective function: area under the linearised demand functions

plus linearised benefit from storage in the terminal period

less total cost of desalination investment and water supply

less total cost of recycling investment and water supply

less total cost of tank investment and water supply

less total cost of trade investment and water supply

less total cost of aquifer investment and water supply

less total cost of new dam investment and water supply

less the marginal operating cost of existing dam water supply

### Demand and convexity constraints

#### Convexity of linearised demands

(B.2)

#### Convexity of linearised demand for terminal storage

(B.3)

#### Balance of the demand for dam water storage in the terminal period

(B.4)

#### Water demand balance

(B.5)

### Existing dam constraints

#### Water supply from existing dams

(B.6)

#### Maximum dam storage capacity

(B.7)

### Desalination constraints

#### Maximum desalination supply capacity

(B.8)

#### Upper bound on total installed desalination capacity

(B.9)

### Recycling constraints

#### Maximum recycling supply capacity

(B.10)

#### Upper bound on total installed recycling capacity

(B.11)

### Tank constraints

#### Maximum tank supply capacity

(B.12)

#### Upper bound on total installed recycling capacity

(B.13)

### Rural–urban interconnection constraints

#### Maximum rural–urban interconnection supply capacity

(B.14)

#### Upper bound on total installed rural–urban interconnection capacity

(B.15)

### Aquifer constraints

#### Maximum aquifer supply capacity

(B.16)

#### Upper bound on total installed aquifer capacity

(B.17)

### New dam constraints

#### Maximum supply capacity of new dams

(B.18)

#### Upper bound on total installed capacity of new dams

(B.19)

## B.2	Policy constraints and variables

### Water restrictions

The water restrictions constraints restrict outdoor water demand when storage is below the trigger level in the preceding year. The restrictions are controlled with the binary variables vRestrYr,N, which have a value of 1 when the restriction is ‘on’, and 0 when the restriction is ‘off’.

#### Water restrictions convexity constraint

(B.20)

#### Water consumption when restrictions are imposed

(B.21)

#### Water restrictions triggered when storage is below threshold

(B.22)

### Long-run marginal cost pricing (with scope for water restrictions)

The long-run marginal cost policy constraints set a uniform price for all demand classes during the regulatory period.

#### Setting uniform prices for all classes of demand during each regulatory period

(B.23)

## B.3	Sets, parameters and variables

Table B.1	Sets in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| AquifCapMap | Yr,N,YrP,NP | Mapping of aquifer investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| AquifCapMaxMap | Yr,N,YrP,NP | Mapping of aquifer investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| d |  | Classes of demand for water |
| DesalCapMap | Yr,N,YrP,NP | Mapping of desalination investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| DesalCapMaxMap | Yr,N,YrP,NP | Mapping of desalination investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| dr | d | Subset of demand which are subject to restrictions |
| FirstYear |  | First year in the simulation period |
| l |  | Linear segments in the demand function |
| LastYrNd | Yr,N | Final year and corresponding node |
| m |  | Linear segments in the function for terminal storage |
| N, NP |  | Nodes in the scenario tree |
| NdPrBlk | Yr,N,PrBlk | Mapping of nodes to uniform pricing blocks |
| NwDamCapMap | Yr,N,YrP,NP | Mapping of new dam investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| NwDamCapMaxMap | Yr,N,YrP,NP | Mapping of new dam investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| PrBlk |  | Uniform pricing blocks |
| RecycCapMap | Yr,N,YrP,NP | Mapping of new recycling investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| RecycCapMaxMap | Yr,N,YrP,NP | Mapping of recycling investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| TanksCapMap | Yr,N,YrP,NP | Mapping of tank investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| TanksCapMaxMap | Yr,N,YrP,NP | Mapping of tank investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| TradeCapMap | Yr,N,YrP,NP | Mapping of rural-urban interconnection investment decisions in Yr and N to the years YrP, NP in which the investment is active, reflecting the lag of investment |
| TradeCapMaxMap | Yr,N,YrP,NP | Mapping of rural-urban interconnection investment decisions in Yr and N to all preceding years in which that investment decision could have been made |
| Yr, YrP |  | Time period. Single years for the shorter planning horizon model, aggregate years for the larger planning horizon model |
| YrNd | Yr,N | Mapping each node to its matching year in the scenario tree |

Source: Productivity Commission urban water model.

Table B.2	Parameters in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| AquifICap | scalar | Total available units of installable aquifers |
| AquifUnitSize | scalar | The size of a unitary investment in aquifers |
| Area | Yr,N,d,l | The probability weighted, discounted welfare from curve d, at node N in Yr, for linear segment l |
| CapExAquif | Yr,N | The probability weighted, discounted and truncated CapEx for aquifers |
| CapExDesal | Yr,N | The probability weighted, discounted and truncated CapEx for desalination |
| CapExNwDam | Yr,N | The probability weighted, discounted and truncated CapEx for new dams |
| CapExRecyc | Yr,N | The probability weighted, discounted and truncated CapEx for recycling |
| CapExTanks | Yr,N | The probability weighted, discounted and truncated CapEx for tanks |
| CapExTrade | Yr,N | The probability weighted, discounted and truncated CapEx for rural-urban interconnection |
| DesalICap | scalar | Total available units of installable desalination |
| DesalUnitSize | scalar | The size of a unitary investment in desalination |
| DPrice | Yr,N,d,l | Retail price of water from curve d, at node N in Yr, for linear segment l |
| EvapLoss | scalar | Proportionate losses from storages in a given year as a result of evaporation |
| InflowAtNode | Yr,N | Level of inflows in period Yr at node N |
| MaxS0 | scalar | Maximum dam storage capacity in existing dams |
| NwDamAdStCap | Yr,N | Additional storage capacity from new dam investment |
| NwDamICap | scalar | Total available units of installable new dams |
| NwDamInflowAtNode | Yr,N | Level of inflows from new dams in period Yr at node N |
| NwDamUnitSize | scalar | The size of a unitary investment in new dams |
| OpexAquif | Yr,N | The probability weighted OpEx (including reticulation costs) for aquifers |
| OpexDam | Yr,N | The probability weighted OpEx (including reticulation costs) for existing dams |
| OpexDesal | Yr,N | The probability weighted OpEx (including reticulation costs) for desalination |
| OpexNwDam | Yr,N | The probability weighted OpEx (including reticulation costs) for new dams |
| OpexRecyc | Yr,N | The probability weighted OpEx (including reticulation costs) for recycling |
| OpexTanks | Yr,N | The probability weighted OpEx (including reticulation costs) for tanks |
| OpexTrade | Yr,N | The probability weighted OpEx (including reticulation costs) for rural-urban interconnection |
| Qty | Yr,N,d,l | Level of water demanded from curve d, at node N in Yr, for linear segment l |
| QtySt | m | Final period storage level linear segment m |
| RecycICap | scalar | Total available units of installable recycling |
| RecycUnitSize | scalar | The size of a unitary investment in recycling |
| Rest | d | Restricted maximum demand for demand type d |
| Rest0 | d | Maximum demand for type d for unrestricted demand (999) |
| S0 | scalar | Water in storage at the start of the simulation period |
| TanksICap | scalar | Total available units of installable tanks |
| TankSupSize | Yr,N | The supply capacity a unitary investment in tanks in period Yr and node N |
| TermStorArea | Yr,N,m | The probability weighted, discounted welfare from terminal storage at node N for linear segment m (including restriction penalty) |
| TradeICap | scalar | Total available units of installable rural-urban interconnection |
| TradeUnitSize | scalar | The size of a unitary investment in rural-urban interconnection |
| Trig |  | Minimum water in storage to trigger restrictions |
| Trig0 | scalar | Minimum water in storage for unrestricted demand |

Source: Productivity Commission urban water model.

Table B.3	Variables in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| Equil | PrBlk | Price equilibration for all prices uniform pricing block PrBlk |
| QAquifCap | Yr,N | Incremental investment in period Yr at node N in aquifers |
| Qd | Yr,N,d,l | Activity variable for linear demand segment l at node N in Yr for demand type d |
| QdDam | Yr,N,m | Activity variable for terminal storage demand segment l at node N in Yr |
| QDesalCap | Yr,N | Incremental investment in period Yr at node N in desalination |
| QNwDamCap | Yr,N | Incremental investment in period Yr at node N in new dams |
| QRecycCap | Yr,N | Incremental investment in period Yr at node N in recycling |
| QsAquif | Yr,N | Quantity of water supplied in period Yr at node N from aquifers |
| QsDam | Yr,N | Quantity of water supplied in period Yr at node N from existing dams |
| QsDesal | Yr,N | Quantity of water supplied in period Yr at node N from desalination |
| QsNwDam | Yr,N | Quantity of water supplied in period Yr at node N from new dams |
| QsRecyc | Yr,N | Quantity of water supplied in period Yr at node N from recycling |
| QsTanks | Yr,N | Quantity of water supplied in period Yr at node N from tanks |
| QstDam | yr,pt | Quantity of water stored at node N in Yr |
| QsTrade | Yr,N | Quantity of water supplied in period Yr at node N from rural-urban interconnection |
| QTanksCap | Yr,N | Incremental investment in period Yr at node N in tanks |
| QTradeCap | Yr,N | Incremental investment in period Yr at node N in rural-urban interconnection |
| vRestr | Yr,N | Binary variable determining if restrictions are active in at node N in Yr |
| vRestr0 | Yr,N | Continuous variable determining if demand is unrestricted at node N in Yr |

Source: Productivity Commission urban water model.