C	Sensitivity testing

This appendix contains details of the sensitivity testing conducted to examine the robustness of the model results to changes in parameter values.

Table C.1 describes each of the sensitivities conducted and the changes to the parameters used relative to the parameters used throughout the rest of this technical supplement (referred to as the ‘central estimate’ values).

Table C.1	Sensitivities examined

|  | Change relative to the central estimate |  |
| --- | --- | --- |
| Sensitivity | Lower bound | Upper bound |
| Mean inflows in to dams | -30% | +30% |
| Price elasticities of demand |  |  |
| Discount rate | -1 percentage point | +1 percentage point |
| Growth rate of demand | -0.5 percentage points | +0.5 percentage points |
| Calibration quantity for the demand curve | -10% | +10% |
| Initial storages in dams | -30% | +30% |

The results contained in this technical supplement come from a very large number of simulations. Table C.2 identifies the different permutations of the model implied by the nature of the analysis.

Table C.2	Potential permutations of the model

| Permutation | Details |
| --- | --- |
| City | Melbourne, Perth |
| Simulation period | Historic, present |
| Simulation timeframe | 10 year horizon, 20 year horizon |
| Basecase/policy models | Basecase; restrictions; investment mandates and bans; two-stage cost minimisation; multi-stage cost minimisation; uniform price |
| Sensitivity testing | Central estimate, mean inflows in to dams, price elasticities of demand, discount rate, growth rate of demand, calibration quantity for the demand curve, initial storages in dams |

Several hundred permutations of the model could have been run. However, solving models of this size (see chapter 3) takes between 6 and 111 hours. It was not practical or necessary to run all permutations identified in table C.2.

Tables C.3 and C.4 contain the results for a subset of the sensitivity tests of parameter values relating to:

- the Melbourne and Perth models

- the historic model

- 10 and 20 year time horizons

- the basecase and policy mandate scenarios

- all sensitivity tests and the central estimate.

Table C.3	Sensitivity of the net social welfare results to changes in parameters for Melbourne

$m

|  | 10 year horizon |  |  | 20 year horizon |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Basecase | Policy | Change | Basecase | Policy | Change |
| Central estimate | 39 371 | 37 393 | 1 978 | 64 945 | 61 469 | 3 476 |
| Low inflows | 38 294 | 36 768 | 1 526 | 63 494 | 60 749 | 2 746 |
| High inflows | 39 853 | 37 699 | 2 154 | 65 443 | 61 764 | 3 679 |
| Low elasticity | 72 916 | 70 952 | 1 964 | 120 534 | 117 062 | 3 472 |
| High elasticity | 22 598 | 20 636 | 1 963 | 37 162 | 33 713 | 3 449 |
| Low discount rate | 40 996 | 39 113 | 1 883 | 120 249 | 116 937 | 3 313 |
| High discount rate | 37 855 | 35 790 | 2 065 | 37 283 | 33 660 | 3 623 |
| Low growth rate | 38 623 | 36 612 | 2 011 | 67 639 | 64 107 | 3 532 |
| High growth rate | 40 134 | 38 191 | 1 943 | 62 455 | 59 043 | 3 412 |
| Low Q point | 35 703 | 33 614 | 2 089 | 63 720 | 60 051 | 3 669 |
| High Q point | 42 963 | 41 131 | 1 832 | 66 202 | 62 994 | 3 208 |
| Low initial storages | 39 192 | 37 297 | 1 895 | 58 898 | 55 556 | 3 342 |
| High initial storages | 39 505 | 37 462 | 2 044 | 70 883 | 67 293 | 3 590 |

Source: Modelling results — Melbourne historic model.

Table C.4	Sensitivity of the net social welfare results to changes in parameters for Perth

$m

|  | 10 year horizon |  |  | 20 year horizon |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Basecase | Policy | Change | Basecase | Policy | Change |
| Central estimate | 22 539 | 22 272 | 267 | 38 681 | 38 148 | 533 |
| Low inflows | 22 313 | 22 065 | 249 | 38 299 | 37 831 | 468 |
| High inflows | 22 687 | 22 405 | 282 | 38 930 | 38 373 | 557 |
| Low elasticity | 41 543 | 41 284 | 258 | 71 360 | 70 836 | 523 |
| High elasticity | 13 041 | 12 768 | 272 | 22 353 | 21 808 | 546 |
| Low discount rate | 23 464 | 23 205 | 259 | 41 707 | 41 172 | 535 |
| High discount rate | 21 674 | 21 400 | 274 | 36 007 | 35 472 | 535 |
| Low growth rate | 22 106 | 21 836 | 270 | 37 335 | 36 789 | 546 |
| High growth rate | 22 980 | 22 717 | 263 | 40 083 | 39 569 | 514 |
| Low Q point | 20 382 | 20 100 | 282 | 34 977 | 34 420 | 557 |
| High Q point | 24 645 | 24 397 | 249 | 42 313 | 41 825 | 488 |
| Low initial storages | 22 509 | 22 247 | 261 | 38 641 | 38 113 | 529 |
| High initial storages | 22 564 | 22 293 | 271 | 38 716 | 38 179 | 537 |

Source: Modelling results — Perth historic model.