A	Mathematics of the model

A complete mathematical specification of the urban water model is presented in this appendix. All variables in the model are identified with names that start with lower case letters. All parameters are identified with names that start with an upper case letter (see tables A.2 and A.3).

All variables in the model are positive (i.e. greater than or equal to zero). Three binary variables (cumpipe, vRestr and vRestrTerm) must take on a value of 0 or 1.

Equations for the core market model, representing scarcity-based pricing are presented first (A.1). Policy interventions are modelled by constraining the core market model according to the equations listed in section A.2.

## A.1	Core market model

### Objective function

(A.1)

Objective function: area under the linearised demand functions less reticulation costs

plus linearised benefit from storage in the terminal period

less total cost of water supply from desalination

less total supply cost from pre-existing and new dam investments

less total supply cost of piping water from rural to urban regions

less total supply costs of household tank water supply

less total cost of water supplied from aquifers

### Dam constraints

#### Maximum dam storage

(A.2)

#### Catchment supply

(A.3)

#### Water demand balance

(A.4)

### Desalination constraints

#### Maximum desalination supply

(A.5)

#### Upper bound on installed desalination capacity

(A.6)

#### Cumulative desalination investment

(A.7)

### New dam constraints

#### Maximum supply for new dams

(A.8)

#### Upper bound new dam capacity

(A.9)

#### Cumulative new dam investment

(A.10)

### Rural–urban trade pipeline constraints

#### Rural–urban trade pipeline supply

(A.11)

#### Upper bound rural–urban trade pipeline capacity

(A.12)

#### Cumulative rural–urban trade pipeline investment

(A.13)

### Household tank constraints

#### Tank supply

(A.14)

#### Cumulative tank investment

(A.15)

### Aquifer constraints

#### Aquifer supply

(A.16)

#### Upper bound aquifer capacity

(A.17)

#### Cumulative aquifer investment

(A.18)

### Constraints related to linearisation

#### Linearised demand for water

(A.19)

#### Linearised benefit function for water stored in the terminal period

(A.20)

#### Demand for storing water in the terminal period

(A.21)

## A.2	Policy constraints and variables

### Cost recovery (without restrictions)

The cost recovery constraints impose a revenue constraint on suppliers of urban water in each time period (for a particular node). This sets total revenue equal to the total cost of supplying water (including investment costs). An additional constraint is imposed to ensure a uniform price for all users of water (within a year and node), which amounts to average cost pricing.

#### Defining total annualised costs of supply (setting the revenue requirement)

(A.22)

#### Written down value of assets in service using a declining balance method

(A.23)

#### Total revenue

(A.24)

#### Cost-recovery (total revenue equals total cost requirement)

(A.25)

#### Uniform price across classes of customers

(A.26)

### Water restrictions

The water restrictions constraints restrict outdoor water demand when storage is below the trigger level in the preceding year. The restrictions are controlled with the binary variables vRestryr,pt,stage, which have a value of 1 when the restriction is ‘on’, and 0 when the restriction is off.

#### Water restrictions

(A.27)

#### Additional term included in the objective function to reflect the cost of restriction after the terminal period

#### Water restrictions in terminal period

(A.28)

#### Water consumption when restrictions are imposed

(A.29)

#### Water restrictions triggered when storage is below threshold

(A.30)

#### Water restrictions triggered when storage is below threshold for terminal period

(A.31)

### Long-run marginal cost pricing (with scope for water restrictions)

The long-run marginal cost policy constraints set a uniform price for all demand classes during the regulatory period (described in figure 4.1). An arbitrarily large term (999) is added to the long-run marginal cost constraints to ensure that when restrictions are active, the level of demand will be determined by restrictions, rather than long-run marginal cost prices.

#### Setting uniform prices for all classes of demand during each regulatory period — upper bound

(A.32)

#### Setting uniform prices for all classes of demand during each regulatory period — lower bound

(A.33)

### Alternative specification of long-run marginal cost pricing with scope for water restrictions

An alternative specification of long-run marginal cost is used as part of the sensitivity analysis (appendix D). This constraint imposes a price of water that is equal to the levelised cost of the next cheapest source of water available to water utilities. Initially rural–urban trade will be the marginal source. Once a pipeline is constructed, desalination is the next cheapest source of water.

#### Setting prices equal to the marginal source — upper bound

(A.34)

#### Setting prices equal to the marginal source — lower bound

(A.35)

## A.3	Parameters, variables and sets

Table A.1	Sets in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| aquilag | 1 | Time lag on aquifer investments |
| d | 1 | Classes of demand for water |
| damlag | 1 | Time lag on new dam investments |
| firstyr | 1 | First year in the simulation period |
| desallag | 1 | Time lag on desalination investments |
| dr | d | Used to apply restrictions to a subset of demands |
| iaq | 1 | Aquifer investment options |
| idm | 1 | New dam investment options |
| ids | 1 | Desalination investment options |
| iru | 1 | Rural-urban pipe investment options |
| itk | 1 | Household tank investment options |
| l | 1 | Linear segments in the demand function |
| lastpt | 1 | Nodes associated with the final year in the probability tree |
| lastyr | 1 | Final year in the simulation period |
| m | 1 | Linear segments in the benefit function for terminal storage |
| pipelag | 1 | Time lag on rural-urban pipe investments |
| PrBlk | 1 | Long-run marginal cost pricing blocks |
| pt, ptp | 1 | Nodes in the decision tree |
| PtPrBlk | pt,PrBlk | Mapping of node pt to a long-run marginal cost pricing block PrBlk |
| stage | 1 | Stage of water restrictions (e.g. stage 3, stage 4) |
| tanklag | 1 | Time lag on household tank investments |
| yr | 1 | Time period. Single years for the shorter planning horizon model, aggregate years for the larger planning horizon model |
| yrpt | yr,pt | Mapping each node pt to its matching year in the probability tree |

Source: Productivity Commission urban water model.

Table A.2	Parameters in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| Aglin | d,yr | Constant for the linear demand function d in yr |
| AreaQd | d,yr,pt,l | Welfare from function curve d, at node pt in yr, for linear segment l |
| AreaSt | lastpt,m | Welfare from final period storage at node lastpt for linear segment m |
| Bglin | d,yr | Coefficient for the linear demand function d in yr |
| Capstock0 | scalar | Initial capital stock in the urban water system |
| DamStCap | idm | Additional dam storage capacity from new dam investment idm |
| Depr | scalar | Depreciation rate |
| Df | yr | Discount factor in yr |
| Discount | scalar | Discount rate |
| FutStorC | lastpt | Welfare cost of restrictions in the period after final node lastpt |
| Gf | yr | Growth factor in yr |
| Inflow_state | pt | Inflow level relative to mean inflows at node pt |
| Level_pipe | yr | Levelised cost of rural-urban pipe water in yr |
| Level-desal | yr | Levelised cost of desalination water in yr |
| OperAqui | iaq | Fixed annual operating cost of aquifer investment iaq |
| OperDam | idm | Fixed annual operating cost of new dam investment idm |
| OperDesal | ids | Fixed annual operating cost of desalination investment ids |
| OperPipe | iru | Fixed annual operating cost of rural-urban pipe investment iru |
| OperSystem | scalar | Fixed annual operating cost of existing water infrastructure |
| OperTank | iru | Fixed annual operating cost of household tanks investment itk |
| PriceQd | d,yr,pt,l | Price of water for users of type d, at node pt in yr, for linear segment l |
| Prob | pt | Probability of being at node pt |
| QtyQd | d,yr,pt,l | Demand water quantity of type d, at node pt in yr, for linear segment l |
| QtyQstdamf | lastpt,m | Final period storage at node lastpt for linear segment m |
| Rest | d,stage | Restricted maximum demand for type d for stage level restrictions |
| Rest0 | d | Maximum demand for type d for unrestricted demand (999) |
| S0 | scalar | Water in storage at the start of the simulation period |
| SwAquiICap | iaq,yr | Water capacity of fully constructed aquifer investment iaq in yr |
| SwDamICap | idm,yr | Water capacity of fully constructed new dam investment idm in yr |
| SwDamStCap | idm,yr | Additional storage capacity from new dam investment idm |
| SwDesalICap | ids,yr | Water capacity of fully constructed desalination investment ids in yr |
| SwEnvFlows | yr | Environmental flows from catchments in yr |
| SwInflows | yr | Mean inflows in yr |
| SwLossFlows | yr | System losses in yr |
| SwMaxS0 | yr | Base storage capacity from initial infrastructure in yr |
| SwPipeICap | iru,yr | Water capacity of fully constructed rural-urban pipe iru in yr |
| SwTankICap | itk,yr | Water capacity of fully constructed household tanks itk in yr |
| TrDamInvC | idm,yr | Truncated new dam investment cost for investment idm in yr |
| TrDesalInvC | ids,yr | Truncated desalination investment cost for investment ids in yr |
| Trig | stage | Minimum water in storage to trigger stage level restrictions |
| Trig0 | scalar | Minimum water in storage for unrestricted demand |

(Continued next page)

Table A.2	(continued)

| Name | Dimensions | Description |
| --- | --- | --- |
| TrPipeInvC | iru,yr | Truncated rural-urban pipe investment cost for investment iru in yr |
| TrTankInvC | iru,yr | Truncated household tank investment cost for investment itk in yr |
| TtrAquiInvC | iaq,yr | Truncated aquifer investment cost for aquifer investment iaq in yr |
| Utcdam | scalar | Unit transport (reticulation) costs of water |
| Vcaqui | scalar | Unit cost of water from aquifers |
| Vcdam | scalar | Unit cost of water from catchments |
| Vcdam2 | scalar | Unit cost of water from new dams |
| Vcdesal | scalar | Unit cost of water from desalination |
| Vcpipe | scalar | Unit cost of water from rural-urban trade |
| Vctank | scalar | Unit cost of water from household tanks |
| WACC | scalar | Weighted average cost of capital |
| YrIndex | yr | Number of years in time period yr |

Source: Productivity Commission urban water model.

Table A.3	Variables in the model

| Name | Dimensions | Description |
| --- | --- | --- |
| assets | yr,pt | Total stock of depreciated assets at node pt in yr |
| costs | yr,pt | Total system costs for cost recovery at node pt in yr |
| cumaqui | iaq,yr,pt | Cumulative proportion of total aquifer investment iaq made at node pt in yr (greater than or equal to 0) |
| cumdam | idm,yr,pt | Cumulative proportion of total new dam investment idm made at node pt in yr (between 0 and 1) |
| cumdesal | ids,yr,pt | Cumulative proportion of total desalination investment ids made at node pt in yr (between 0 and 1) |
| cumpipe | iru,yr,pt | Cumulative proportion of total rural-urban pipe investment iru made at node pt in yr (binary variable 0 or 1) |
| cumtank | iru,yr,pt | Cumulative proportion of total household tanks investment itk made at node pt in yr (greater than or equal to 0) |
| equil | d,PrBlk | Price equilibration for all prices of demand type d in long-run marginal cost pricing block PrBlk |
| NW | scalar. | Net social quasi-welfare |
| onePrice | yr,pt | Uniform price for cost recovery |
| qsaqui | yr,pt | Quantity of water supplied from aquifers at node pt in yr |
| qsdam | yr,pt | Quantity of water from pre-existing dams at node pt in yr |
| qsdam2 | yr,pt | Quantity of water supplied from new dams at node pt in yr |
| qsdesal | yr,pt | Quantity of water supplied from desalination at node pt in yr |
| qspipe | yr,pt | Quantity of water supplied from rural-urban trade at node pt in yr |
| qstank | yr,pt | Quantity of water supplied from household tanks at node pt in yr |
| qstdam | yr,pt | Quantity of water stored at node pt in yr |
| revenue | yr,pt | Total revenues received by the authority for cost recovery at node pt in yr |
| vRestr | yr,pt,stage | Binary variable determining if restrictions of stage are active in the at node pt in yr |
| vRestr0 | yr,pt | Continuous variable determining if demand is unrestricted at node pt in yr |
| vRestrTerm | lastyr,lastpt,stage | Binary variable determining if restrictions of stage would be active in the next year after node lastpoint in lastyr |
| vRestrTerm0 | lastyr,lastpt | Continuous variable determining if demand would be unrestricted in the next year after node lastpoint in lastyr |
| wQd | d,yr,pt,l | Weight or activity level for linear demand segment l at node pt in yr for demand type d |
| wQstdamf | lastpt,m | Weight or activity level for terminal storage linear segment m in at final-year node lastpt |

Source: Productivity Commission urban water model.